HAZARDOUS WASTES

RCRA of 1976:

  - Defines hazardous waste

  - Promotes recovery and conservation

  - Mandates government control of ultimate disposal from point of generation to point of disposal (cradle-to-grave concept)

  - Requires manifest identification and permitting system

  - Amended to require double liners and leachate collection system

  - Applies to generators producing at least 220 lb. in a calendar month

CERCLA of 1980, amended in 1986:

  - Defines hazardous substances as used in the Clean Air Act and Clean Water Act

  - Regulates leachate and other releases from abandoned and inactive sites operating prior to November 1980

  - Businesses that produce 220–2,000 lb of hazardous wastes in a calendar month are also regulated

  - Comes into play when hazardous waste sites are identified and classified

  - Established priorities list of sites targeted for remediation (bad first)

  - Gov’t can require owner/operator to clean up site

  - Established “Superfund” via tax on chemical production

  - If responsible party cannot be found, gov’t can perform clean up with state contribution of 10% of cleanup cost

Superfund Amendments and Reauthorization Act (SARA) of 1986:

  - First mandatory gov’t program requiring chemical emergency planning at the state and local level

  - Extension of CERCLA that established remediation standards and increased funding to implement the program

  - Title III of SARA:

    - Emergency Planning and Community Right to Know Act (EPCRA) established

    - Provides public with knowledge about chemical hazards in their area

Toxic Substances Control Act of 1976:

  - Regulates production, use, and disposal of chemical substances that may present an unreasonable risk of injury to health or environment

Clean Air Act (EPA):

  - Regulates emission of hazardous air pollutants

Clean Water Act (EPA):

  - Regulates discharge of hazardous pollutants into the nation’s waters

Marine Protection, Research, and Sanctuaries Act (EPA):

  - Regulates waste disposal at sea

Occupational Safety and Health Act (OSHA):

  - Regulates hazards in the work place, including worker exposure to hazardous substances

  - Requires toxic waste operators to have 40 hours of training and protective equipment (HAZWOPER)

Hazardous Materials Transportation Act (DOT):

  - Regulates transportation of hazardous materials

Hazardous waste defined:

Solid waste or combination of solid wastes that because of its quantity, concentration, or physical, chemical, or infectious characteristics may:

  - Cause or significantly contribute to an increase in mortality or an increase in serious irreversible or incapacitating reversible illness

  - Pose a substantial present or potential hazard to human health or the environment when improperly treated, stored, transported, or disposed of or otherwise managed

Includes the following substances:

  - Chemical

  - Biological

  - Flammable

  - Explosive

  - Radioactive

Characteristics:

  - Ignitability

  - Corrosivity

  - Reactivity/explosiveness

  - Toxicity (identified via toxicity characteristics leaching procedure, TCLP)

Hazardous waste recycling information required:

  - Company ID code

  - Category

  - Primary usable constituents

  - Contaminants

  - Physical state

  - Quantity

  - Packaging

  - Geographic location

Transportation:

  - CHEMTREC offers info and advice on what to do and on characteristics of chemicals in an emergency…available 24 hours a day, 365 days a year

  - In case of fire or other emergency at a facility having hazardous materials, info can be obtained via the National Oceanic and Atmospheric Administration (NOAA)

  - Information availability is required by the Superfund Amendments and Reauthorizatin Act (SARA)

Long-term storage:

  - Double liners with leachate collection systems required

  - Groundwater monitoring system required

  - Can be assumed that all liners, cutoff walls, or other containments will eventually leak

Manifest system:

  - Links generator, transporters, storage facilities, treatment plants, disposal sites, and the EPA/state

  - If incident occurs, hazards/risks can be recognized and responsible parties identified

Shipping (all on hazardous waste manifest):

  - Proper shipping name

    - Chemical name

    - Chemical group/family

    - End use

    - Hazard class

  - UN/NA Number

  - Hazard class (DOT)

    - Explosives

    - Gases

    - Flammable and combustible liquids

    - Flammable solids

    - Oxidizers

    - Poisonous, toxic, and infectious liquids/solids

    - Radioactive

    - Corrosive

    - Miscellaneous

  - Packing group

    - I = Great danger

    - II = Medium

    - III = Minor

  - Markings labels and placards

    - Orientation arrows

    - Hazardous waste label (and enter required waste and DOT markings on label)

    - Hazard label (DOT) and place primary hazard DOT label next to waste label

  - Additional information (handling)

Treatment technologies:

  - Physical

  - Chemical

  - Biological

  - Thermal

- g/ppm = density of vapor for HAZWOPER

- Hazardous materials drained from crank case = heavy metals, oil

- Process modification = zero tolerance for hazardous waste

- Corrosion-resistant container = best for storing hazardous waste

- Radioactive waste management = DOE (Dept. of Energy)

- Storage of hazardous waste without permit = 90 days max