RESIDENTIAL AND INSTITUTIONAL ENVIRONMENT

Facts

- Every family and individual has a basic right to a decent home and a suitable living environment.

Realization of a decent home in a suitable living environment requires:

  - Clean air

  - Pure water and food

  - Adequate shelter

  - Unpolluted land

  - Freedom from excessive noise and odors

  - Adequate recreational and neighborhood facilities

  - Convenient community services in an environment that provides safety, comfort, and privacy

- APHA (American Public Health Association) appraisal method for measuring housing quality was developed by the Committee on the Hygiene of Housing between 1944 and 1950.

APHA appraisal method:

  - Attempts to minimize/eliminate individual opinion to arrive at numerical value

  - May be compared with results in other cities

  - Grouped under Facilities, Maintenance, and Occupancy

  - Rating is based on a penalty system

  - Theoretical maximum score is 600

  - Score of zero indicates a perfect score

  - Score of 200 or greater indicates unfit conditions

Conditions constituting basic deficiency include:

  - Over 1.5 persons per room

  - Number of occupants equals or exceeds 2 times the number of sleeping rooms plus 2

  - Less than 40 square feet of sleeping area per person (crowding)

  - Installed bath lacking, shared with other dwelling unit, or outside structure

  - Water supply outside dwelling unit

  - Outside window lacking in any room unit

  - No installed electric lighting

- A trained Sanitarian can inspect about 10 dwelling units per day.  For every 4 inspectors, there should be one trained field supervisor, three office clerks, and one office supervisor.

Environmental sanitation and hygiene indices:

  - Cost estimates for rehabilitation of selected buildings and foot survey (re-inspect) should be made to confirm administrative judgment and decisions before any major action is taken

NEEDS (Neighborhood Environmental Evaluation and Decision System)

  - 5 stages

  - Provides rapid and reliable measure of neighborhood quality

Basic health principles of housing and its environment:

  - Living unit and structure

    - Human factors

    - Sanitation and maintenance

    - Safety and injury prevention

  - Residential environment

    - Community of individual facilities

    - Quality of the environment

    - Environmental control programs

Outline of a housing program:

  - Establishment of a committee(s)

  - Identification of the problem

  - Informing the community

  - Designation of a board or commission

  - Appropriation of adequate funds

  - Appraisal and designation of areas

  - Preparation and adoption of an enforceable housing code and other regulations

  - Institution of a systematic and planned code enforcement program

  - Concurrent provision and upgrading

  - Provision of new and rehabilitated housing units

  - Aid in securing financial and technical assistance

  - Evaluation of progress made and continual adjustment

Enforcement procedure:

  - Education and persuasion

    - New releases and informational bulletins

    - Inspection

    - Notification

    - Re-inspection

    - Second notification

    - Third notification

  - Legal action as last resort

    - Informational hearings

    - Administrative hearings

    - Pre-trial hearings

    - Summonses

Structural safety:

  - Building must be able to support two and half to four times the loads and stresses to which it is or may be subjected

  - Water-repellent floor with sanitary cove base or its equivalent is necessary

  - Must have window or skylight not less than 10% of the floor area, with at least 45% “openable” providing adequate light and ventilation

  - Ventilation system may be approved in lieu of a window or skylight

- Every room occupied for sleeping purposes shall contain at least 70 square feet of floor space for one person and 50 square feet for each additional person.

- For backflow prevention, terminate the water supply inlet or faucet with air gap of twice the diameter of the pipe above the flood-level rim of the fixture.

- Indirect waste piping should discharge to an open-water supplies sink or receptacle so that the end of the waste pipe terminates at least 2 inches above the rim of the sink or receptacle, which is directly connected to the drainage system.

- Most urban dwellers spend as much as 80–90% of their time indoors, including transportation vehicles.

Primary types in indoor air quality problems are:

  - Inadequate ventilation (52%)

  - Contamination from inside the building (17%)

  - Contamination from outside the building (11%)

  - Microbiological contamination (5%)

  - Contamination from building fabrics (3%)

  - Unknown (12%)

Sick building syndrome:

  - Eliminate VOCs

  - Up HVAC air exchange

  - Eliminate allergens and mold

- A study showed a 45% increase in respiratory infection in energy-efficient buildings.

- Young children, the elderly, and people suffering from respiratory diseases will be the first to show signs of discomfort from indoor air contamination.

Radon (and its alpha-emitting decay products):

  - Major portion of the biologically-significant dose associated with natural background radiation

  - Hazard association with radon is related to concentration and time of exposure

  - Should not exceed 2–5 pCi/l indoors

  - Contamination in existing dwelling can be reduced by preventing entry or removing

  - Can be reduced by

    - Closing and caulking all cracks, joints, and openings of the structure in the basement or in contact with the ground or in the flooring above the crawl space

    - Tightly covering open drains and sumps

Formaldehyde:

  - Colorless gas

  - May cause extreme discomfort and contact dermatitis indoors

  - Odor can be detected at less than 1 ppm

  - Sources are

    - Resins and glues to bond particle board and plywood

    - Urea-formaldehyde foam insulation

    - Permanent press fabric

    - Embalming fluid

    - Drugs

    - Disinfectants

    - Cosmetics

    - Chemicals used in pathology and anatomy laboratories

    - Chemicals used in the manufacture of automobiles

    - Furniture

    - Paper

    - Electrical equipment

    - Beauty products in a cosmetology salon (hair treatments, etc…)

    - Materials used in mobile homes and prefabricated housing

  - Minimum ventilation of 5 air changes per hour required

Thermal and moisture requirements:

  - EPA recommends relative humidity of 30–50% for homes

  - Indoor relative humidity of 60% or higher would cause excessive condensation and greater mildew, corrosion, and decay

  - Humidity above 70% promote germination and growth of fungal spores

Venting of heating units:

  - The high moisture in vent gases condenses inside a chimney forming sulfuric acid, which attacks lime in the mortar, leaching it out, and creating leaks and destruction of the chimney

  - Flue/vent extended at least 3 ft above flat roofs or 2 ft above the highest part of the wall parapets and peaked roof ridges will be reasonably free from downdrafts

  - An opening of 100–200 square inches will usually provide sufficient fresh air under ordinary household conditions

  - Connection between the furnace or stove and chimney should be tight fitting and slope up to the chimney at least ¼ in/ft

Mobile home parks:

  - Mobile homes are “transportable, single-family dwelling units”

  - “Manufactured relocatable living unit”

  - Minimum spacing of 10 feet required

Institutions:

  - Communities unto themselves

  - Possibilities for transmission of illnesses associated with air, water, food, and contact are increased

  - Provide ideal environment for spread of communicable diseases

  - Schools should allow 25–30 square feet per student classroom space

  - If a school suspects radon, ground-level classrooms should be monitored

Hospitals:

  - Nosocomial infection = hospital-acquired infection

    - Urinary tract is most frequent site of infection

    - Contributing factors are

      - Older patients with chronic disease

      - Increase in high-risk patients and surgical procedures such as organ transplants and open-heart surgery

      - Innovations in diagnostic and therapeutic procedures

      - Inadequate disinfection or sterilization of respiratory therapy and other equipment

    - Prevention measures include

      - Hygienic medical, nursing, and staff practices (e.g., hand washing)

      - Equipment sterilization

      - Food, water, plumbing, air, laundering, linen handling, and housekeeping sanitation

      - Prevention of overcrowding

      - Minimization of movement of patients and hospital personnel from point to point

      - Avoidance of antibiotic use when possible

      - Establishment of a representative infection control committee

  - Hospital laundry wash-water temperatures must be a minimum of 160 to 167◦F for 25 minutes

Hospital wastes:

  - Vacuum cleaners are problematic…cause particulates in air

  - Only about 15% of all hospital wastes are infectious

  - Infectious and pathological wastes (e.g., disposable needles, syringes, scalpels) are best disposed of via incineration and autoclaving followed by compaction or shredding

    - Recommendations to equal or exceed air quality standards include temperature of 2,000◦F with 2 seconds residence time and secondary chamber temperature of 1,800◦F

  - Chemotherapy and pathological wastes are incinerated

  - Autoclaved wastes are disposed of in a landfill or incinerator

  - General hospital and kitchen wastes can usually be disposed of through the community waste collection system

  - Liquid infectious wastes may be carefully poured to a drain connected to a sanitary sewer

  - The usual method for the disposal of low-level radioactive solid waste is by storage until decayed, followed by disposal with the general waste

Prisons:

  - Known causes of prison unrest and illness include

    - Food poisoning

    - Poor and insufficient food

    - Vermin infestation

    - Inadequate work and recreational programs

    - Overcrowding

  - Should allow for 60 square feet per person

  - Prisoners required to shower twice per week

  - Prisoners required to change clothes twice per week

  - Prisoners should be fed twice per day

  - A prison is a complete community

  - Prisoners should be guaranteed adequate recreation and work

  - Max water temp for hand washing in jail is 120 F

Day care centers:

  - Enteric and respiratory disease transmission via the fecal-oral route and by intimate contact is more common among children in a day-care center

  - Multiple pathogen infection is not uncommon

  - Diarrhea in day cares can be prevented by hygiene, hand washing, food sanitation, separation of ill children, education of staff and management in disease transmission and prevention