WATER TREATMENT (incomplete)

Disinfection:

- Chlorination is most common method

- Effectiveness of chlorine is dependent on:

  - Water pH

  - Temperature

  - Contact time

  - Water clarity

- pH should be 8.0 or less

- Average monthly MPN of coliform bacteria should be less than 50/100

- Recommended field tests for residual chlorine are the DPD colorimetric and SNORT methods

  - High concentrations of iron and manganese and dirty glassware can cause interference with residual chlorine readings

- Chemical coagulation of water or of wastewater that has received biological treatment can remove 99% of viruses

- Diatomaceous earth filtration can remove 98% of viruses, especially if water is pretreated

- Activated carbon is NOT suitable for virus removal

- Minimum free chlorine residual at distant points in the distribution system should be 0.2–0.5 mg/l

- Presence of ammonia, organic matter, and other chlorine-consuming materials requires a higher dosage of chlorine = high chlorine demand

Plain sedimentation is the settling or storage of water, such as would take place in a reservoir, lake, or basin, without the aid of chemicals, preferably for a month or longer, particularly if the source water is a sewage-polluted river water.

Coagulation, flocculation, and settling:

- Coagulants (e.g., alum) permits particles to come together and results in formation of floc and attract materials in settling out

- It is recommended that mixing tanks and settling basins be at least 2 in number to permit cleaning and repairs w/out interrupting treatment

Filtration:

- Primary purpose is to remove suspended materials

- Slow sand filters are recommended for use in small communities

- Diatomaceous earth filters are commonly used for industrial water and swimming pool water and NOT recommended for drinking water

Water treatment plant wastewater and sludge:

- Required by Clean Air Act to be adequately treated prior to discharge to a surface water course

- Common treatment processes include:

  - Drying beds

  - Lagoons

  - Freezing and thawing (natural)

  - Dewatering (chemical and/or mechanical)

- May be disposed of by:

  - Lagooning

  - Discharge to wastewater treatment plant

  - Mechanical dewatering and landfilling

    - During free residual chlorination, trihalomethanes (THMs) may be formed, which are suspected of being carcinogenic.

Granular activated carbon filters:

- Used for treating water for soft drinks and bottled drinking water

- Works via adsorption

- Used in reservoirs and settling basins to exclude sunlight causing growth of algae (a.k.a., blackout treatment)

- Good for odor and taste removal from drinking water

Reservoir management, intake control, and stratification

- Classifications:

  - Eutrophic (productive)

  - Oligotrophic (unproductive)

- Temperature fluctuations (winter and spring when 39.2 degrees F is reached) can cause “turnover”

- Stratification into 3 layers:

  - Epilimnion

    - Top mixed zone

    - High in oxygen and algae

    - Contains euphotic zone and trophogenic region

  - Metalimnion

    - Middle transition zone

    - Often the source of the best water

  - Hypolimnion

    - Bottom zone of stagnation

    - Often deficient in oxygen

    - Contains tropholytic region

    - Pumps should be of capacity to deliver average daily water demand to the storage tank in 6–12 hrs.

Pumping stations should be at least 3 ft above the 100-yr flood plain of the highest know level, whichever is higher.

It is recommended that water storage equal not less than 1/2 the total daily consumption, with at least 1/2 the storage in elevated tanks.